erosion and pollution control notes

- All contractors shall comply with the north central texas council of governments (nctcog) manual for best management practices for construction as it relates to preventing pollutants in storm water discharge from construction activities.

- The contractor shall install erosion and pollution control measures as indicated on the plans and as field conditions warrant to prevent offsite migration of soils or other pollutants by vehicular tracking or in storm water runoff.  Prior to the commencement of any construction activity, installation of control measures, repairs or modifications to the measures will be made by the contractor if the control measures prove ineffective or if additional control measures are necessary.

- The contractor shall install the stabilized construction exit and shall take appropriate measures to prevent tracking of mud and/or soils to offsite pavement.  The contractor shall immediately remove any offsite tracking that occurs.

- All stockpiled soils will be surrounded by a silt fence, sediment control swale or equivalent measure to properly control sediment runoff.

- The contractor shall designate a concrete truck washout area.  Waste concrete and associated water and material will be properly disposed of in accordance with the applicable regulations.

- The contractor shall construct a berm or other spill protection measure for any temporary fuel storage tank(s) on site TO CONTAIN AT LEAST ONE HALF (1/2) OF THE CAPACITY OF EACH TANK.

- If sump pumps are used to remove water from excavated areas, the discharge shall be filtered or discharged to a settling basin to remove sediment and other pollutants before the water enters a storm drain or leaves the site.

- Inlet protection shall be installed at all onsite or new inlets to prevent sediment and soils from entering the storm drain system UNTIL PERMANENT VEGETATION IS ESTABLISHED.  Inlet protection shall be designed to filter all flow from a two-year storm event.  A means of overflow or bypass shall be provided for storm flows greater than the two-year storm.

- Control measures may only be placed in front of inlets or in channels, drainage ways or borrow ditches at the contractor’s risk.  Contractor shall remain liable for any damage caused by the measure including flood damage that occurs as a result of blocked drainage.

- At a minimum, perimeter controls such as silt fence shall be installed at all down slope boundaries of disturbed areas and as warranted where pavement removal, utility construction, grading or other construction activities are to be performed.  The contractor shall at all times take such measures as necessary to prevent offsite migration of sediment and debris.

- If discharge of soil or other pollutants occurs, the control measures shall be evaluated and changes or additional measures implemented within seven (7) days to prevent future discharges.

- Damage to adjacent property and/or to receiving water caused by improperly installed or poorly maintained erosion and pollution control measures will be the responsibility of the contractor.  This responsibility includes removal and disposal of any siltation, debris or other pollutants caused by his/her operations and/or failure of the control measures.  fAILURE TO ADDRESS THE CAUSES OF DAMAGES WILL RESULT IN A STOP WORK ORDER.

- The contractor shall remove and properly dispose of accumulated sediment before it reaches a depth of six (6) inches or impairs the effectiveness of the control measure.

- The owner’s representative will inspect the project every fourteen (14) days, at a minimum, and after every rain event of one-half inch or greater to determine the integrity and effectiveness of the erosion and pollution control measures.  A written inspection report, signed by the person making the inspection, will be filed with onsite pollution control plan and a copy of the report sent to the city for their records.  This inspection does not relieve the contractor of responsibility for inspection and maintenance of the control measures.  fAILURE TO FILE REPORTS MAY RESULT IN A STOP WORK ORDER.

- The contractor shall stabilize, with some form of ground cover, any area where construction activity is to be ceased (temporarily or permanently) for more than twenty-one (21) days.

- At the conclusion of the project, all channels, drainage way and borrow ditches in the work zone shall be cleared of any sediment and debris generated by the project or deposited as a result of the erosion and pollution control measures.

- Revegetation of all disturbed soil shall be initiated within twenty-one (21) days of final construction operations.